Mathematical Derivation FSDT

Free vibration analysis of compressor blade having NACA thickness, camber, taper, pre-twist, stagger angle and is rotating at given RPM (rotation effects) and the root of blade is at a distance from the axis. Using FSDT and Rayleigh-Ritz method. To find frequency and mode shape. The mathematical derivations are:

# Geometric Definition

The analysis begins with the position vector r0 of a point on the middle surface of the cambered, tapered, and pre-twisted blade. Adopting a counter-clockwise twist convention, the position vector is expressed as:

r0(x, β)=(x+Rhub) i+βcos⁡Φx-ζcsin⁡Φxj+βsin⁡Φx+ζccos⁡Φxk

where:

- Φx=ϕstagger+θ0x

- dΦxdx=θ0.  θ0 is the twist rate,

# FSDT Displacement Field

The fundamental assumption of the First-Order Shear Deformation Theory (FSDT) is that a straight line initially normal to the middle surface remains straight after deformation, but it does not necessarily remain perpendicular to the surface. This relaxation introduces two additional independent degrees of freedom, representing the rotations of the normals.

The total displacement of a generic point located at a distance ζ from the middle surface is described by five primary displacement fields:

- u0(x, β,t)Middle-surface displacement in the spanwise (x) direction.

- v0xβt: Middle-surface displacement in the chordwise (β) direction.

- w0xβt: Transverse displacement in the normal (ζ) direction.

- ψxxβt: Independent rotation of the normal in the x-ζ plane.

- ψβxβt: Independent rotation of the normal in the β-ζ plane.

The displacement components at any point xβζ are given by the following kinematic relationships:

- uxβζt=u0xβt+ζ ψxxβt

- vxβζt=v0xβt+ζ ψβxβt

- wxβζt=w0xβt

# FSDT Strain–Displacement Relationships

The internal state of the blade is described by the linear strain components. Under FSDT, the total strain at any point is decomposed into membrane, bending, and transverse shear components.

## Membrane Strains (ϵ0)

The membrane strains describe the stretching and in-plane shearing of the middle surface. Unlike simplified plate theories, these include curvature terms 1R which provide the geometric coupling essential for cambered blades

ϵ110=1A∂u0∂x+v0AB∂A∂β+w0Rx
ϵ220=1B∂v0∂β+u0AB∂B∂x+w0Ry
γxβ0=1A∂v0∂x-u0AB∂A∂β+1B∂u0∂β-v0AB∂B∂x+2w0Rxβ

where A and B are the derived Lamé parameters, and Rx, Ry, Rxβ are the principal and twist radii of curvature.

## Bending Strains (Curvature Changes, κ)

### Drilling Rotation (ϖ)

The drilling rotation, denoted ϖ, represents the in-plane spin of the blade surface. It serves as a crucial kinematic coupling between in-plane displacements (u, v) and out-of-plane bending in geometrically twisted configurations. It is defined as:

ϖ=12AB∂∂xBv-∂∂βAu

Applying the quotient rule-consistent with derivations in the reference literature-this expression can be expanded to the equivalent form:

ϖ=121A∂v∂x-1B∂u∂β+vAB∂B∂x-uAB∂A∂β

### Bending Strain Measures (Curvature Changes, κ)

Within the framework of First-Order Shear Deformation Theory (FSDT), the bending strains are derived from the gradients of the independent rotational fields (ψx, ψβ) and the drilling rotation ϖ. These curvature terms are essential for capturing the geometric stiffening effects characteristic of wide-chord compressor blades.

Spanwise bending strain (κxx):

κxx=1A∂ψx∂x+ψβAB∂A∂β+ϖRxβ

Chordwise bending strain (κββ):

κββ=1B∂ψβ∂β+ψxAB∂B∂x-ϖRxβ

Twist bending strain (κxβ):

κxβ=121A∂ψβ∂x-ψxAB∂A∂β+1B∂ψx∂β-ψβAB∂B∂x-1Rx-1Ryϖ

These formulations are mathematically consistent and physically appropriate for the doubly curved, twisted geometry under consideration.

### Transverse Shear Strains (γ)

These are the critical additional terms provided by FSDT. They represent the shear deformation through the thickness of the airfoil. This flexibility allows the NACA arch to "breathe" under load, mimicking the behavior of 3D solid elements:

γxz=ψx+1A∂w0∂x-u0Rx-v0Rxβ
γβz=ψβ+1B∂w0∂β-v0Ry-u0Rxβ

### Total Through-Thickness Strains

The total normal and shear strains at any distance ζ from the middle surface are then expressed as:

ϵ11=ϵ110+ζ κxx
ϵ22=ϵ220+ζ κββ
γxβ=γxβ0+2ζ κxβ

# Energy Functionals for Rotating FSDT Shells

The total system energy is the sum of the internal strain energy, kinetic energy, and potential energy from centrifugal effects.

## Strain Energy (Us)

The total strain energy is obtained by integrating the work done by internal stresses over the volume of the blade. It is decomposed into three primary components: membrane, bending, and shear:

Us=120L0bxϵ0TDmϵ0+κTDbκ+γTDsγAB dβ dx

where:

- Membrane Strain Vector, ϵ0=ϵ110ϵ220γxβ0

- Bending Strain Vector, κ=κxxκββ2κxβ

- Transverse Shear Strain Vector, γ=γxzγβz

- Transposed Transverse Shear Strain Vector, γT=γxzγβz

- Membrane Stiffness Matrix, Dm=E h1-ν21ν0ν10001-ν2

- Bending Stiffness Matrix, Db=E h3121-ν21ν0ν10001-ν2

- Transverse Shear Stiffness Matrix, Ds=ks E h21+ν1001 or equivalently, using the shear modulus G: Ds=ks G h1001, where G=E21+ν

Important Notes for Implementation:

- h=h(x,β) — thickness varies with both spanwise and chordwise coordinates (due to NACA profile and taper), so it must remain inside the double integral.

- ks — shear correction factor (≈ 0.822 for rectangular cross-sections).

- E — Young's modulus (may be temperature-dependent if thermal effects are included).

- ν — Poisson's ratio.

- All three matrices are symmetric and constant with respect to the curvilinear coordinates xβ, except for the explicit h and h3 scaling.

Kinetic Energy (T):

## Translational and Rotary Inertia (T1)

1. Fundamental Energy Definition

The kinetic energy component associated with the relative velocity within the rotating frame is:

T1=12ρVr⋅r dV

where r=u i+v j+w k is the velocity relative to the rotating frame.

Expanding the dot product:

T1=12ρV(u2+v2+w2 dV

2. FSDT Displacement Field

Under First-Order Shear Deformation Theory (FSDT), the 3D displacement field varies linearly through the thickness ζ:

| Component | Expression | Description |
| --- | --- | --- |
| Spanwise | uxβζ=u0xβ+ζψxxβ | In-plane stretching + bending rotation about β-axis |
| Chordwise | vxβζ=v0xβ+ζψβxβ | In-plane stretching + bending rotation about x-axis |
| Transverse | wxβζ=w0xβ | Constant through thickness (no thickness stretching) |

Where:

- u0,v0,w0 = mid-surface translational displacements

- ψx,ψβ = rotations of the cross-section normal

- ζ = through-thickness coordinate, ζ∈-h2,h2

3. Through-Thickness Integration

Substitute the FSDT displacement field into the kinetic energy integrand:

LT1=12ρ-h/2h/2u0+ζψx2+v0+ζψβ2+w02dζ

Expand the squares:

LT1=12ρ-h/2h/2u02+2ζu0ψx+ζ2ψx2+v02+2ζv0ψβ+ζ2ψβ2+w02dζ

Standard Through-Thickness Integrals

Since displacements are referenced to the geometric middle surface:

-h/2h/2dζ=h
-h/2h/2ζ dζ=0 (cross-terms vanish)
-h/2h/2ζ2 dζ=h312

After Integration

The cross-terms (linear in ζ) vanish, leaving:

LT1=12ρhu02+v02+w02+h312ψx2+ψβ2

4. Mid-Surface Energy Density

The energy density per unit mid-surface area is:

LT1=12ρhu02+v02+w02+h312ψx2+ψβ2

Physical Interpretation

| Term | Expression | Physical Meaning |
| --- | --- | --- |
| Translational | hu02+v02+w02 | Mass per unit area × velocities² — standard mass matrix |
| Rotary | h312ψx2+ψβ2 | Rotary inertia — cross-sectional warping inertia |

5. Global Integration for a Tapered Blade

To obtain the total energy, integrate over the blade surface with Lamé parameters A and B:

T1=12ρ0L-ptbx1-ptbxhxβu02+v02+w02+hxβ312ψx2+ψβ2AB dβ dx

- First term: Translational kinetic energy of the mid-surface displacements.

- Second term: Rotary inertia from the independent rotation fields (cross-sectional warping inertia).

- h=h(x,β) remains inside the integral to account for variable thickness.

- The dot symbol() used in kinetic energy derivation represents a partial derivative with respect to time. So: u02=iω *u0x,β, t2=-ω2*u0x,β, t2

## Gyroscopic (Coriolis) Coupling (T2)

- Fundamental Energy Definition

For a compressor blade rotating at constant angular velocity Ω=Ωj about the global Cartesian Y-axis, the kinetic energy cross-term between relative vibration velocity rrel and frame rotation velocity Ω×Rdeformed is given by:

T2=ρVrrel⋅Ω×Rdeformed dV

The position vector of a deformed point at a normal distance ζ∈-h/2h/2 from the middle surface is:

Rdeformed=X0+ζnx+uCartesiani+Y0+ζny+vCartesianj+Z0+ζnz+wCartesiank

where r0=X0i+Y0j+Z0k is the middle-surface position vector, a3=nxi+nyj+nzk is the unit normal vector, and uCartesian,vCartesian,wCartesian are the FSDT displacement components along global Cartesian axes.

- 3D Cross-Product & Volume Integrand

Evaluating the cross-product Ω×Rdeformed for rotation about the Y-axis:

Ω×Rdeformed=ΩZ0+ζnz+wCartesiani-ΩX0+ζnx+uCartesiank

Taking the inner product with the relative velocity vector rrel=uCartesiani+vCartesianj+wCartesiank gives:

IT2=ρΩuCartesianZ0+ζnz-wCartesianX0+ζnx+ρΩuCartesianwCartesian-wCartesianuCartesian

- Variational Order Isolation

The integrand splits into two variational terms:

- Rigid-Body Static Term:

ρΩuCartesianZ0+ζnz-wCartesianX0+ζnx

This term is linear in generalized velocity qit with coefficients independent of generalized displacement qi. After volume integration, it collapses to a total time derivative ddticiqit with constant coefficients ci, which vanishes identically under Euler–Lagrange equations:

ddt∂T2rigid∂qi-∂T2rigid∂qi=ddtci-0=0

- Bilinear Dynamic Gyroscopic Term (T2):

ρΩuCartesianwCartesian-wCartesianuCartesian

This term is bilinear in dynamic velocity and displacement Oqq, making it second-order in perturbation Oδ2—identical in dynamic order to kinetic energy Oq2 and strain energy Oq2. This term represents the active gyroscopic energy coupling.

- Kinematic Projections

Projecting the FSDT displacement field u=u0+ζψxa1+v0+ζψβa2+w0a3 onto global Cartesian axes perpendicular to the rotation axis:

uCartesian=F0+ζFψ  ⟹  uCartesian=F0+ζFψ
wCartesian=H0+ζHψ  ⟹  wCartesian=H0+ζHψ

where the spatial kinematic functions and their time derivatives are:

F0=u0A+w0nx, F0=u0A+w0nx
Fψ=ψxA, Fψ=ψxA
H0=u0A∂Z0∂x+v0B∂Z0∂β+w0nz, H0=u0A∂Z0∂x+v0B∂Z0∂β+w0nz
Hψ=ψxA∂Z0∂x+ψβB∂Z0∂β, Hψ=ψxA∂Z0∂x+ψβB∂Z0∂β

- Through-Thickness Integration

Substituting the kinematic expressions into the bilinear integrand gives:

uCartesianwCartesian-wCartesianuCartesian=F0H0-H0F0+ζF0Hψ-HψF0+FψH0-H0Fψ+ζ2FψHψ-HψFψ

Integrating through the thickness ζ∈-h/2h/2:

-h/2h/21 dζ=hxβ, -h/2h/2ζ dζ=0, -h/2h/2ζ2 dζ=hxβ312

The linear ζ-term vanishes because displacements are referenced to the geometric middle surface, leaving the mid-surface energy density:

UT2=ρΩhF0H0-H0F0+ρΩh312FψHψ-HψFψ

- Final Expression for Gyroscopic Energy (T2)

Integrating over the blade surface with the surface metric Jacobian AB dβ dx:

T2=ρΩ0LβLEβTEhxβF0H0-H0F0+hxβ312FψHψ-HψFψAB dβ dx

## Centrifugal Softening (rotational softening potential energy, T3)

- Undeformed Geometry and Kinematic Reference Frame

For a blade rotating about the global Y-axis with angular velocity Ω=Ωj, the undeformed middle surface is described by curvilinear coordinates xβ.

The unified chord-orientation angle is:

Φx=ϕstagger+θ0x

The position vector of a point on the undeformed middle surface is:

r0xβ=X0i+Y0j+Z0k

where:

X0=x+Rhub

Y0=βcosΦx-ζcxβsinΦx

Z0=βsinΦx+ζcxβcosΦx

- Lamé Parameters and Basis Vectors

The exact Lamé parameters are:

A=1+βθ0+∂ζc∂x2+ζc2θ02 ;B=1+∂ζc∂β2

The covariant unit tangent vectors and unit normal vector are:

a1=1A∂r0∂x, a2=1B∂r0∂β, a3=a1×a2

- FSDT Displacement Field

Under FSDT, the 3D displacement vector at normal distance ζ∈-h2h2 is:

u=u0+ζψxa1+v0+ζψβa2+w0a3

Projecting onto global Cartesian axes perpendicular to the rotation axis:

uCartesian=u⋅i, wCartesian=u⋅k

Using the exact dot products:

a1⋅i=1A, a2⋅i=0, a3⋅i=nx
a1⋅k=1A∂Z0∂x, a2⋅k=1B∂Z0∂β, a3⋅k=nz

The exact kinematic projections are:

uCartesian=u0+ζψxA+w0nx
wCartesian=u0+ζψxA∂Z0∂x+v0+ζψβB∂Z0∂β+w0nz

- Centrifugal Potential Energy

The centrifugal potential energy is:

T3=12ρΩ2VXdeformed2+Zdeformed2dV

The softening terms (quadratic in displacements) are isolated as:

Isoftening=uCartesian2+wCartesian2

Define the compact quantities:

F0=u0A+w0nx, Fψ=ψxA
H0=u0A∂Z0∂x+v0B∂Z0∂β+w0nz
Hψ=ψxA∂Z0∂x+ψβB∂Z0∂β

The softening integrand becomes:

Isoftening=F02+H02+2ζF0Fψ+H0Hψ+ζ2Fψ2+Hψ2

- Through-Thickness Integration

Integrating through the thickness ζ∈-h2,h2:

-h/2h/21 dζ=h, -h/2h/2ζ dζ=0, -h/2h/2ζ2 dζ=h312

The linear ζ-term vanishes, yielding the exact softening energy density:

Usoftening=12ρΩ2hF02+H02+h312Fψ2+Hψ2

- Final Expression

Integrating over the blade surface with the true surface Jacobian AB:

T3softening=12ρΩ20LβLEβTEhF02+H02+h312Fψ2+Hψ2AB dβ dx

where:

F0=u0A+w0nx, Fψ=ψxA
H0=u0A∂Z0∂x+v0B∂Z0∂β+w0nz
Hψ=ψxA∂Z0∂x+ψβB∂Z0∂β

## Theoretical Derivation of Initial Stress Potential Energy (Uσ) for Centrifugal Stiffening

- Fundamental Energy Functional

The strain energy stored in a doubly curved FSDT shell due to the complete 2D initial in-plane stress field Nxx0Nββ0Nxβ0is:

Uσ=120LβLEβTENxx0ϵxxNL+Nββ0ϵββNL+2Nxβ0ϵxβNLAB dβ dx

where A and B are the middle-surface Lamé parameters, and ϵxxNL,ϵββNL,ϵxβNLare the non-linear Green–Lagrange strain components.

- Complete Non-Linear FSDT Strain Components

Under First-Order Shear Deformation Theory (FSDT), the non-linear strain components driving geometric stiffening across all three translational displacement fields u0v0w0are:

ϵxxNL=121A∂u0∂x2+1A∂v0∂x2+1A∂w0∂x2
ϵββNL=121B∂u0∂β2+1B∂v0∂β2+1B∂w0∂β2
ϵxβNL=121A∂u0∂x1B∂u0∂β+1A∂v0∂x1B∂v0∂β+1A∂w0∂x1B∂w0∂β

- Initial Stress Resultants

- Spanwise Normal Stress Resultant (Nxx0)

The total axial centrifugal tension at spanwise station xis the cumulative mass-acceleration outboard of x:

Faxialx=xLρAsectionx'Ω2Rhub+x' dx'

where the local cross-sectional area is:

Asectionx'=βLEβTEhx'βBx'β dβ

Distributing Faxialxproportionally across the local thickness hxβ:

Nxx0xβ=FaxialxAsectionxhxβ

Boundary check: Nxx0is nonzero at both chordwise edges, which is correct — the spanwise tension is carried across the full chord by the membrane, and the free edges are free only in the chordwise direction.

- Chordwise Normal Stress Resultant (Nββ0)

The chordwise centrifugal pull is generated by the component of centrifugal acceleration projected along the chordwise tangent, proportional to Ω2sin2Φx, where Φx=ϕstagger+θ0x.

Physical boundary condition: Both chordwise edges are free (unsupported), so the chordwise stress resultant must vanish identically at both ends:

Nββ0xβLE=0, Nββ0xβTE=0

Correct formulation: Integrate outward from the point of interest β to whichever free edge is nearer.

Nββ0xβ=ρΩ2sin2Φxββnearest edgehxβ'Bxβ' dβ'

where:

βnearest edge=βLE,β<βcenterβTE,β≥βcenter

and βcenteris the chordwise location of the neutral axis of chordwise tension (the pre-twist axis, taken as β=0 if the stacking line is at mid-chord).

- In-Plane Shear Stress Resultant (Nxβ0)

Due to pre-twist θ0and camber variation Zcη, differential centrifugal pull across the chord creates a static spanwise twisting moment Mtwist0x:

Mtwist0x=xLβLEβTEρhx'β'Ω2Rhub+x'β'sinΦx'+ζcx'β'cosΦx'Bx'β' dβ' dx'

A compressor blade cross-section is a solid / open FSDT shell. The model St. Venant open-section torsion, characterized by the torsional constant:

Jtorsionx=13βLEβTEhxβ3Bxβ dβ

The resulting static in-plane shear stress resultant is:

Nxβ0xβ=Mtwist0xJtorsionx hxβ2

- Un-Truncated Potential Energy Functional (Uσ)

Substituting the three non-linear strain expressions into the surface energy integral yields the exact, un-truncated potential energy functional:

Uσ=120LβLEβTE{ Nxx01A∂u0∂x2+1A∂v0∂x2+1A∂w0∂x2+  Nββ01B∂u0∂β2+1B∂v0∂β2+1B∂w0∂β2+  2Nxβ01A∂u0∂x1B∂u0∂β+1A∂v0∂x1B∂v0∂β+1A∂w0∂x1B∂w0∂β} AB dβ dx

# Ritz Displacement Functions and Orthogonal Polynomials

To transform the continuous structural problem into a discrete system of algebraic equations, the five displacement and rotation fields are represented as a finite summation of orthogonal polynomials.

## Ritz Expansion

The displacement components and normal rotations are expanded as follows:

u0xβt=m=1Mn=1NUmn Tmξ Tnη eiωt

v0xβt=m=1Mn=1NVmn Tmξ Tnη eiωt

w0xβt=m=1Mn=1NWmn Tmξ Tnη eiωt

ψxxβt=m=1Mn=1NΨx,mn Tmξ Tnη eiωt

ψβxβt=m=1Mn=1NΨβ,mn Tmξ Tnη eiωt

where:

- Umn,Vmn,Wmn,Ψx,mn,Ψβ,mn are the unknown coefficients (Ritz parameters);

- Tmξ and Tnη are the orthogonal basis functions in the spanwise and chordwise directions, respectively;

- ξ=2xL-1 and η=2βbx are the non-dimensional coordinates.

## Construction of Basis Functions

In the Rayleigh-Ritz discretization of the governing equations of motion, the five primary displacement fields-spanwise displacement u0, chordwise displacement v0, transverse displacement w0, and the normal rotations ψx and ψβ are approximated using series of orthogonal polynomials. Each field is expressed as:

Φξηt=m=1Mn=1NamnTmξTnηeiωt

where amn denote the unknown Ritz coefficients (assembled into the global vector X), and Tmξ and Tnη are the orthogonal basis functions in the spanwise and chordwise directions, respectively.

### Coordinate Transformation

To enable efficient numerical integration via Gauss–Legendre quadrature, the physical geometry of the pre-twisted, tapered compressor blade is mapped onto a standardized computational domain.

- Spanwise Direction (ξ):

The physical spanwise coordinate x∈0L is transformed to ξ∈-11 via:

ξ=2xL-1

- Chordwise Direction (η):

The normalized chordwise coordinate η∈01 measures the distance from the leading edge (η=0) to the trailing edge (η=1):

ηxy=ybx

where y∈0bx is the physical chordwise coordinate, and bx is the local tapered chord width.

- Singularity-Removing Quadratic Coordinate Transformation for NACA Thickness

To evaluate the FSDT stiffness and mass energy integrals via Gauss–Legendre quadrature, the physical geometry of the blade is mapped onto a standardized computational domain.

For a NACA 4-digit airfoil profile, the standard thickness distribution hη over the non-dimensional chordwise coordinate η∈01 (where η=0 is the leading edge and η=1 is the trailing edge) is defined as:

hη=2bxtmax0.20.2969η-0.1260η-0.3516η2+0.2843η3-0.1015η4
Differentiating hη with respect to η yields an infinite slope at the leading edge:

limη→0dhdη∝limη→00.29692η→∞
Because the FSDT energy functionals integrate hxβ and h3xβ over the blade surface, this infinite gradient makes the integrands non-smooth at η=0. When evaluated using standard linear Gauss–Legendre quadrature, this non-smoothness breaks exponential convergence, causing slow algebraic convergence in modes with high leading-edge strain energy (such as Mode 1).

To remove this mathematical singularity and restore exponential quadrature convergence, a quadratic coordinate substitution is introduced:

η=t2, for t∈01

Substituting η=t2 into the thickness distribution transforms the leading square-root term into an exact linear function of t:

η=t2=t
ht2=2bxtmax0.20.2969t-0.1260t2-0.3516t4+0.2843t6-0.1015t8

The transformed function ht2 is a smooth, analytic polynomial in t over the entire domain 01.

The differential mapping relationship (Jacobian of the substitution) is:

dη=2t dt

To perform quadrature over the standard interval η∈-11, the auxiliary variable t∈01 is mapped via:

tη=12η+1, dt=12 dη

Consequently, any chordwise integral featuring the NACA thickness distribution is transformed as:

01Fη dη=01Ft2⋅2t dt=12-11Ftη2⋅2tη dη

The Jacobian factor 2tη analytically cancels the 1/η singularity at the leading edge (t→0), enabling exponential Gauss–Legendre quadrature convergence at low integration point counts.

### Enforcement of Geometric Boundary Conditions

In First-Order Shear Deformation Theory (FSDT), the transverse displacement w and the cross-sectional rotations ψx,ψy are independent kinematic fields. For a cantilevered compressor blade, the essential (geometric) boundary conditions at the clamped root (x=0, i.e., χ=-1) require that all translational displacements and independent rotations vanish:

u0=0, v0=0, w0=0, ψx=0, ψy=0 at χ=-1.

A key feature of FSDT is that transverse shear deformation is accounted for, so the mid-surface slope ∂w0/∂x is not required to be zero at the root. Instead, the boundary condition is satisfied solely by enforcing w0=0 and ψx=0 independently.

To impose these essential boundary conditions, the first polynomial in the spanwise series, X1χ, is chosen with a linear factor that vanishes at the root:

X1χ=χ+1.

In the spanwise direction (χ), for a clamped-free configuration, the root factor χ+11 ensures that all trial functions are exactly zero at the fixed end (χ=-1), thereby satisfying the geometric constraints without overly restricting the slope or inducing boundary shear locking.

In the chordwise direction (η), both edges are free, so no geometric constraints are imposed (α=0). Consequently, the orthogonal polynomials start from a constant function:

Y1η=1.

### Generation of Higher-Order Terms via Gram–Schmidt Process

To ensure numerical stability and accelerate convergence, higher-order terms (k≥2) are generated using a recurrence relation that enforces orthogonality over the domain:

T2χ=χ-B1T1χ
Tk+1χ=χ-BkTkχ-CkTk-1χ, k≥2

The recurrence coefficients Bk and Ck are computed from the following integrals:

Bk=χaχbχ Tkχ2 dχχaχb[Tkχ]2 dχ, 
Ck=χaχbχ Tkχ Tk-1χ dχχaχb[Tk-1χ]2 dχ

# State-Space Formulation and Eigenvalue Extraction

The discrete governing equations of motion for the rotating compressor blade are obtained by applying the Rayleigh–Ritz method to the energy functionals.

MX+GX+K1+Kc-KT+KsprX=0

where:

- Global Ritz Coefficient Vector (X): Vector of unknown weights for basis functions approximating the five displacement fields u0,v0,w0,ψx,ψβ in the Rayleigh-Ritz method.

- Mass Matrix (M): Symmetric positive-definite matrix derived from T1 containing translational inertia ρh and rotary inertia ρh3/12 terms.

- Gyroscopic (Coriolis) Matrix (G): Skew-symmetric matrix derived from T2 coupling relative vibration velocities with rotation speed Ω, causing frequency splitting with increasing RPM.

- Structural Stiffness Matrix (K₁): Symmetric positive-definite matrix derived from strain energy U using Golden'veizer relations with Lamé parameters A,B and curvatures Rx,Rβ,Rxβ.

- Geometric Stiffness Matrix (Kc): Symmetric matrix from Uσ representing centrifugal stiffening due to initial stress resultants from static rotation solution.

- Rotational Softening Matrix (KT): Subtractive symmetric matrix from T3 representing stiffness reduction as deflected mass moves farther from rotation axis.

- Artificial Spring Stiffness (Kspr): Diagonal penalty matrix used in Rayleigh-Ritz to enforce cantilevered boundary conditions at the blade root.

- Quadratic Eigenvalue Problem: -ω2M+iωG+K1+Kc-KT+KsprX=0 solved for natural frequencies ω and mode shapes X.

## Linearization into State-Space

Since the equation of motion contains the first-order derivative term GX, it cannot be solved by standard symmetric eigenvalue solvers. To linearize the problem, we define a state vector Z=XXT. The second-order system is then converted into a first-order state-space representation:

Z=AlinZ

The linearized state-space matrix Alin is constructed as:

Alin=0I-M-1Ktotal-M-1G

## Complex Eigenvalue Solution

Assuming a solution of the form Z=Z0eλt, the problem reduces to a standard eigenvalue problem:

Alin-λIZ0=0

Due to the presence of the unsymmetric G matrix, the eigenvalues (λ) are generally complex. The stability and vibrational characteristics are extracted as follows:

Natural Frequency: The circular frequency is determined from the imaginary part of the complex eigenvalue:

ω=Imλ

Cyclic Frequency: The frequency in Hertz is given by:

f=ω2π